AC System on  Dump truck

Air condition system is one of the major system in the truck . Brief illustration of the  system as below:

Requirement of Air Conditioning System:

No of persons inside the cabin : 2

Brief functioning of Air Conditioning System:

AC Systems are much like a mother, who uses a sponge to soak  up milk from a spilled glass. She soaks it up and then wrings the milk out of the sponge and into the sink. Similarly, an AC system  soaks up heat in the cab using evaporation and then squeezes the heat into the outside air using condensation.

Air conditioners have the expansion valve system with five basic components

1. Expansion device: A restriction in the liquid line of the system, purposely designed to cause a pressure drop.

2. Evaporator: A device that removes heat from the cab air by exchanging it into boiling refrigerant.

3. Compressor: Provides the mechanical energy to move refrigerant and manipulate pressures. This is the heart of the system.

4. Condenser: Designed to exchange heat from the refrigerant to the outside air. Similar to the evaporator.

5. Drier Filter Device: A storage container for extra refrigerant that usually contains a drying agent called

desiccant and a filter to screen out contaminants.

EXPANSION VALVE SYSTEM:

Low-Side Operation:

Cold refrigerant absorbs heat from the cab air while the refrigerant circulates inside the evaporator coil. Heat from the cab air passes through the metal of the evaporator and cause the refrigerant to expand by boiling off into a vapor. This boiling is possible because refrigerants used in AC systems have a boiling point of about one pound per square inch per 1°F. For example, the boiling point of refrigerant is approximately 20°F at 20 psi. As the refrigerant expands by boiling into a vapor, it takes massive amounts of heat with it.

High-Side Operation:

The hot, high pressure vapor makes its next stop at the condensing coil. The condenser is just like the evaporator – it is a heat exchanger. It looks a little different from the evaporator because it is more flat and a little larger. Inside the condensing coil, the gas starts its way from the top to the bottom, cooling down a little with each pass. By the time the refrigerant reaches the lower third of the coil, it cools down enough to change back into a liquid. As a liquid, it continues to cool 15-30°F below its boiling point in a process called “subcooling.”

BTU: 28000 BTU/HR                     
AIR Flow: 320 CFM (540 C.M/Hr) WEIGHT: 47 lbs/ 21 Kgs        MOTORS: Three speed-double shaft motor with 2 blowers           CURRENT DRAWN: 8.9 Amps @ 27.2 DC for the blower motor plus 2 amps for compressor clutch   CONTROLS: AC switch, fan speed switch, fresh air control switch   Refrigerent: R-134a

TOTAL QUANTITY OF REFRIGERENT: 1.2 Kgs and 1.5 Kgs

REQUIREMENTS: 1. CABIN VOLUME : 3478871200 mm3 2. NO OF PERSONS IN THE CABIN : 2 AC Systems are much like a mother, who uses a sponge to soak up milk from a spilled glass. She soaks it up and then wrings the milk out of the sponge and into the sink. Similarly, an AC system soaks up heat in the cab using evaporation and then squeezes the heat into the outside air using condensation.

Air conditioners have two basic layouts: the expansion valve system and the accumulator system. AC primarily uses the expansion valve system With five basic components

1. Expansion device: A restriction in the liquid line of the system, purposely designed to cause a pressure drop.

2. Evaporator: A device that removes heat from the cab air by exchanging it into boiling refrigerant.

3. Compressor: Provides the mechanical energy to move refrigerant and manipulate pressures. This is the heart of the system.

4. Condenser: Designed to exchange heat from the refrigerant to the outside air. Similar to the evaporator.

5. Drier Filter Device: A storage container for extra refrigerant that usually contains a drying agent called desiccant and a filter to screen out contaminants.