Annexure-C

SCOPE OF WORK

MANUFACTURE AND SUPPLY OF TC- ROOF ASSEMBLY FOR MEMU-3P PROJECT

- Brief Scope of Work & Quantity :

| Sl No. | Part No. | Part Descrn | Qty/ Car | Total Quantity Reqd for 78 Cars | Scope of Work |
| --- | --- | --- | --- | --- | --- |
|  | ROOF ASSEMBLY |  |  |  |  |
| 1 | ET16100SC | ROOF ASSY | 1 | 78 | Supply of Fabricated TC Roof Assembly with one coat of Red oxide Zinc Chromate Primer (IS:2074-92) on Roof Assembly and Intermediate Parts. |

LIST OF SUBASSEMBLIES TO BE SUPPLIED BY VENDOR ALONG WITH ROOF ASSEMLY IN LOOSE CONDITION:

| SI.NO | PART NO | DESCRIPTION | UOM | QTY/ CAR | Total Qty Required for 78 Cars |
| --- | --- | --- | --- | --- | --- |
| 1 | CC16431 | Roof Ventilator Grill | NO | 10 | 780 |

Note :

- Rake Formation: 1 Rake- 2DMC+2TC1+2TC2+2TC3

- Vendor should be approved source of RDSO/ICF/RCF

- Manufacture & Supply of Aggregates should conform to Latest drawings and to the material specification/Standards/Test, indicated in the respective drawings is to be adhered/ensured.

- Firm to supply aggregates for one TC Car to ascertain the fitment/installation aspects before bulk supply.

II :  DETAILED  SCOPE OF WORK FOR ROOF ASSEMBLY :

- Details of  TC Roof Assy

- Roof Consists of following  Major Sub Assemblies:

- Roof Structure

- Roof Sheet Assembly

- Manufacturing of Detailed Components:

- All Components to be only manufactured by CNC Flame Cutting/Plasma Cutting/Laser Cutting/Shearing for Plates & Sheets, Saw Cutting for Tubes/Pipes/Bars/Rods. Vendor shall submit nesting plan to BEML for Approval before taking up for manufacturing.

- Dress, Grind and Deburr cut edges of all detailed components and all edge preparation to be carried out by beveling only.

- Straitening of Components to be carried out where ever required before taking up for forming/Machining/Assembly.

- All Plasma cut/Flame cut/Laser cut components to be shot blasted/Cold Phosphated before taking up for forming/Machining/Assembly. All Parts/Members shall be first degrease and Paint with Weld Primer.

- Forming of Parts to be done by reducing Tool Marks.

- Machining of Components to be carried out as per drawing requirement. All machined surfaces to be protected from rusting by applying rust preventive oils and physical damages if any.

- Mass production of detailed components can be taken up only after getting acceptance from BEML for first Sample / Proto Assy Clearance.

- Fabrication of Subassemblies/Assemblies:

- All subassemblies/Assemblies to be fabricated out of weld jigs in order to achieve uniformity in dimensions, quality of above production. This Jigs to be shown to BEML/SIT during manufacturing Proto and subsequent production.

- Welding of Fabrication Structures to be carried out as specified in drawings.

- Welding shall be carried out uniformly along the length to avoid distortion.  During Fabrication, Weld defects, Weld spatters, heat marks etc due to welding shall be removed before taking up for next assembly. Welder should be qualified for the requirements of IS: 7310 and IS: 817 or any equivalent international standards.

- Pickling and Passivation to be carried out for removing weld marks on Stainless Steel surfaces.

- Fabricate all major Sub Assemblies viz, Roof Sheet Assembly and Roof Structure etc., as per Drawings.

- Dimensional check in accordance with the check sheet shall be carried for all major Sub-assemblies before taking for Major Assembly and report to be submitted to BEML.

- Holes, Machined, Stainless Steel & Earthing Surfaces to be Masked and ensure to avoid entry of foreign materials during Cleaning and Painting.

- Cleaning of the Surface of the Roof Assembly with Pressurized Air/Emery paper to remove dust & Debris.

- Loading of Sub assemblies viz, Roof Structure and Roof Sheet required for Roof Assembly on Main Jig Assembly as per drawing. After Full Welding of Roof Sheet Assembly with Roof Structure, install other details Like Ring Complete, Supports and Brackets as specified in drawing.

- Dimensional check in accordance with the check sheet shall be carried for Roof Assembly and report to be submitted to BEML.

- Roof Center Line to be marked/Visible along and across (Inside and Outer Side) on all Roof Assemblies.

- Check the Condition of Masked Areas &Painting Equipment before starting of painting.

- Roof Assembly and Intermediate parts to be painted with one coat of Red oxide Zinc Chromate Primer to IS: 2074-92 Except Stainless Steel Components.

- All other dimensions / standards / specifications to be maintained / followed for all Subassemblies / Assemblies as per respective Drawings with latest alterations. Before starting the production, vendor should ensure to manufacture as per the latest drawing as and when it is updated

- Inspection & Clearance :

- Proto Inspection :

- Offer the   Sub Assemblies Viz, Roof Structure, Roof Sheet Assembly etc, in Tack Welded Condition to BEML/SIT/RITES inspection.

- Offer Roof assembly to BEML/SIT/RITES Inspection in Jig Loading stage of Roof structure, Tack Welded Condition, after installation of Roof sheet assembly on Roof Structure.

- Offer the completed Proto Roof Assembly to BEML/SIT/RITES inspection. Associate with BEML/SIT/RITES Inspection team for final inspection & clearance. Firm shall carryout any modifications suggested by BEML/SIT/RITES Inspection team without any additional cost.

b. General Inspection Conditions :

- Offer all Roof assemblies to BEML/SIT/RITES inspection. Associate with BEML/SIT/RITES Inspection team for final inspection & clearance and firm shall carryout any modifications suggested by BEML/SIT/RITES Inspection team.

- Chemical and Mechanical testing of samples should be carried out as and when insisted by BEML/SIT/RITES inspection team at their discretion.

- The Construction throughout Rood Assembly shall be absolutely Leak Proof. All Roof Skin welded joints shall be tested after Roof Assembly by Applying Chalk Powder on one side and Kerosene oil on other side. Dye Penetration Test to be carried out for all Roof weld joints as and when insisted by BEML/SIT/RITES inspection team at their discretion.

- BEML/SIT/RITES at their sole discretion may inspect any item at any stage of Manufacturing of Roof Assembly.

- The Vendor shall maintain & provide a copy of test reports, inspection reports and Check sheets to BEML. All the witnessed inspection reports like kerosene oil leak test, dye penetrate testing, chemical & Mechanical testing, Dimensional report etc. should be submitted to BEML in digital form via email or CD/DVD.

IV. Delivery Schedule :

- Proto TC Roof Assembly shall be delivered to BEML at RC-II Unit/ Bangalore Complex, after Inspection Clearance & after implementing any modifications if any suggested by BEML/SIT/RITES.

- After Successful Installation of Proto Roof Assembly at BEML, Bulk Production to be taken up after obtaining Clearance from BEML  Inspection Team.

- TC Roof  Assemblies - 12 Cars/Month (After Proto Clearance).

- All aggregates to be supplied as a set in Car wise using proper holding Jigs to RC-II Unit, KGF/Bangalore Complex based on confirmation by M/s.BEML.

V. Material Scope:

- BEML Scope :

- Raw materials in the form of Plates/Sheets.

- Rounds and Tubes in full Lengths.

- Bought Out Items as mentioned at SI.No.VII

- Applicable Drawings.

- Inspection Check Sheet/Inspection Test Plan for  Roof Structure & Roof  Assembly etc.,

- Vendor Scope.

- All required welding consumables, Rust Preventive oils, Lubricants as specified in Drawing/RDSO Specifications.

- Red oxide Zinc Chromate Primer (IS: 2074-92 ) to be procured from ICF/RCF/RDSO Approved sources only, to be inspected by RITES at Source and test certificates to be submitted to M/s.BEML.

- Material Specification of Welding consumables, Rust Preventive Oils, Derusting/ Degreasing/Cold Phosphate consumables & paints and its expiry details shall be furnished to BEML for approval before manufacturing of the first proto Roof Assembly.

VI. General Notes to Comply:

- Quality Assurance Plan (QAP) should be submitted by firm for approval.

- Process Qualification Record (PQR)/Welding Procedure Specification (WPS) should be available.

- Vendor should be ISO: 9001 – 2015 Compliant.

- Before taking up manufacturing activity, Self-Inspect the M/s BEML supplied component for transit damages, In case of any defects same has to be communicated with detailed report and send back the same to BEML in as is condition.

- Vendor shall submit nesting plan to BEML for Approval before taking up for manufacturing. Off cuts and Scrap generated during manufacturing of components to be returned to BEML as per approved nesting plan.

- Ensure components supplied by M/s.BEML and Fabricates Subassemblies and Assemblies are stored properly and damages due to poor & improper storage, workmanship etc. should be the responsibility of Vendor.

- Vendor shall ensure the proper fitment of aggregates while shell integration/furnishing at initial batches of supply & further in case of any fitment issues at RC-II unit KGF and Bangalore Complex. Vendor has to engage their representative for necessary correction and implementation of those corrections in further supplies.

- Any minor scope Changes/Modifications in drawing or during inspection if any to be taken up by vendor without any additional cost.

- All Roof assemblies to be properly protected to avoid damages during loading, unloading and transportation.

VII. LIST OF BOUGHT OUT ITEMS UNDER BEML SCOPE:

| SI.NO | PART NO | PART DESCRIPTION | UOM | QTY/CAR |
| --- | --- | --- | --- | --- |
| 1 | ET16104 | CANTRAIL PART | NO | 2 |
| 2 | ET16109 | CANTRAIL PART | NO | 2 |
| 3 | ET16129 | CANTRAIL PART | NO | 2 |
| 4 | ET16130 | CANTRAIL PART | NO | 2 |
| 5 | ET16143 | CANTRAIL PART | NO | 4 |
| 6 | CFN0110652 | HEX. NUT M6 | NO | 80 |
| 7 | 84921137 | EARTHING BLOCK M8 | NO | 36 |
| 8 | 84931001 | CLAMP ASSY DIA 25 (TRIPLE) | NO | 80 |
| 9 | 84931002 | CLAMP ASSY. DIA 40(2 CONDUITS) | NO | 20 |
| 10 | 94331002 | CLAMP ASSY. DIA 40 | NO | 20 |
| 11 | 94331003 | CLAMP ASSY. DIA 29 | NO | 74 |
| 12 | 94356230 | CLAMP ASSY. DIA 40 | NO | 42 |
| 13 | ICFSK71512C5 | EARTHING LUG | NO | 12 |