Technical mandatory qualification sheet for NDT-UT Service contract during the year 2020-21

Name of Firm:

Place:

| S.No. | Description / Parameters | Agree / Disagree | Supplier comments if any | Remarks |
| --- | --- | --- | --- | --- |
| 01 | Ultrasonic testing to be carried out for: |  |  |  |
|  | 1.1 Plates 12mm and above |  |  |  |
|  | 1.2 Structural welded joints for Main frames |  |  |  |
|  | 1.3 Casting & Forging on specified areas. |  |  |  |
| 02 | Personnel Qualification |  |  |  |
|  | 2.1 Person shall be qualified in ASNT UT LEVEL II - having minimum 2 (two) years relevant experience is required. |  |  |  |
|  | 2.2 Person shall give disposition based on interpretation and evaluation |  |  |  |
| 03 | Working hours: |  |  |  |
|  | 3.1 Shall be 8hrs (9.00am to 5.00pm) – Excluding sunday |  |  |  |
|  | 3.2 Alternate arrangement shall be made in case of absence. |  |  |  |
|  | 3.3 In case of production emergency the firm shall be arranged for man power in holidays. |  |  |  |
| 04 | EQUIPMENT & ACCESSORIES: to be positioned by testing agency. |  |  |  |
|  | 4.1 With Calibrated MODSONIC Digital UT flaw detector (EINSTEIN II TFT) or equivalent with calibration blocks |  |  |  |
|  | 4.2 IIW V1 / V2 block |  |  |  |
|  | 4.3 Flat bottom hole / SDH block (optional) |  |  |  |
|  | 4.4 Step wedge block |  |  |  |
|  | 4.5 Relevant Probes: |  |  |  |
|  | 4.5.1 Angle (45°/60°/70°) with Frequency range of 2 to 4 MHz # each one |  |  |  |
|  | 4.5.2 Normal probe with Frequency range of 2 to 4 MHz & size is min.Dia 5mm / 10mm / 24mm # each one |  |  |  |
|  | 4.5.3 TR probe with Frequency range of 2 MHz / 4MHz # each one |  |  |  |
|  | 4.6 With requisite cables for all probes ( Shall be positioned 3 cables min.) |  |  |  |
| 05 | After UT evaluation & interpretation, detailed report with disposition shall be submitted in standard format. |  |  |  |
| 06 | Firm shall bring basic mobile phone (without camera & data connectivity) for communication |  |  |  |

Authorised signatory