TENDER REF: 6300029826

PART A – TECHNICAL SPECIFICATIONS & CONDITIONS

Description  :   UPGRADING RECORDING CAPACITY CCTV SYSTEM

Qty:  01  No.

Eligibility Criteria:

- The OEM or the Authorized dealer are eligible for participating in the tender.

- The authorized dealer should enclose the duly valid authorization letter along with Technical Bid

Technical   Specifications:

Material Description:

- Supply and installation of NVR, capable/supporting the following:-

- 256 Channel, 768 Mbps, HIKVISION, ACTi, ARECONT, AXIS, BOSCH, BRICKCOM, CANON, HUNT, ONVIF (Version 2.5), PANASONIC, PELCO, PSIA, RTSP, SAMSUNG, SONY, VIVOTEK, ZAVIO

- Minimum two independent HDMI outputs of 4K resolution.

- Minimum 24 SATA interfaces supporting hot-plug, Up to 10 TB for each HDD.

- VCA detection alarm including - Face Detection, Line Crossing, Behavior Analysis, People Counting.

- Video & Audio decoding:

Format- H.265, H.265+, H.264, H.264+, MPEG4, MJPEG

Resolution - 12MP/8MP/7MP/6MP/5MP/4MP/3MP/1080p/UXGA/720p/

VGA/4CIF/DCIF/2CIF/CIF/QCIF

- Video wall output/input 6/1 HDMI.

- The supplied machine and accessories (HDDs, A/C etc) should be installed and fully integrated with the existing CCTV System on Turn-Key basis.

Existing System Details:

# There are 85 nos of existing CCTV Cameras.

# All the existing CCTV Cameras are connected to the CCTV All-in-One Server Model: Hikvision Blazer Pro/128/16H.

# Work for integrating additional 43 cameras to the existing CCTV System is under progress.

Notes  :

- Delivery:   The Equipment   to be delivered to BEML, Bangalore Complex.

.

- Warranty:   Firm should provide warranty for a period of 24-months from the date of successful commissioning

- During Warranty period, the firm should attend to any problem/breakdown of the machines with necessary spares/tools and rectify the problem/breakdown within 24-hours of complaint by the user. If the repair takes more than 24-hrs, the firm should provide suitable replacement for the defective part/machine until it is fully repaired and reinstalled

- The firm should furnish comparative statement / Deviation  Statement of our tender enquiry  for  description,   specifications,  and  notes  vis-à-vis the model offered by them along with the offer.

- Inspection:  the equipment with accessories will be subject to acceptance by our divisional quality-engineering department at BEML Site.

# Compliance to Technical specifications and Technical terms & conditions as per Annexure I [Mandatory]should be uploaded in SRM portal in the Technical bid.