TENDER REF: 6300029826

ANNEXURE I – COMPLIANCE TO TECHNICAL SPECIFICATIONS & CONDITIONS

Description  :   :   UPGRADING RECORDING CAPACITY CCTV SYSTEM

| TECHNICAL INPUTS FOR DESIGN OF THE SYSTEM |  | COMPLIED / NOT COMPLIED / REMARKS |
| --- | --- | --- |
| 1 | Supply and installation of NVR, capable/supporting the following:- |  |
| 2 | 256 Channel, 768 Mbps, HIKVISION, ACTi, ARECONT, AXIS, BOSCH, BRICKCOM, CANON, HUNT, ONVIF (Version 2.5), PANASONIC, PELCO, PSIA, RTSP, SAMSUNG, SONY, VIVOTEK, ZAVIO |  |
| 3 | Minimum 24 SATA interfaces supporting hot-plug, Up to 10 TB for each HDD. |  |
| 4 | VCA detection alarm including - Face Detection, Line Crossing, Behavior Analysis, People Counting. |  |
| 5 | Video & Audio decoding: Format- H.265, H.265+, H.264, H.264+, MPEG4, MJPEG |  |
|  | Resolution - 12MP/8MP/7MP/6MP/5MP/4MP/3MP/1080p/UXGA/720p/ VGA/4CIF/DCIF/2CIF/CIF/QCIF |  |
| 6 | Video wall output/input 6/1 HDMI. |  |
| 7 | The supplied machine and accessories (HDDs, A/C etc) should be installed and fully integrated with the existing CCTV System on Turn-Key basis. |  |

Technical Terms & Conditions:

| Sl. No. | Clause | COMPLIED / NOT COMPLIED / REMARKS |
| --- | --- | --- |
| 1 | - Delivery: The Equipment to be delivered to BEML, Bangalore Complex. |  |
| 2 | - Warranty: Firm should provide warranty for a period of 24-months from the date of successful commissioning |  |
| 3 | - During Warranty period, the firm should attend to any problem/breakdown of the machines with necessary spares/tools and rectify the problem/breakdown within 24-hours of complaint by the user. If the repair takes more than 24-hrs, the firm should provide suitable replacement for the defective part/machine until it is fully repaired and reinstalled |  |
| 4 | The firm should furnish comparative statement / Deviation Statement of our tender enquiry for description, specifications, and notes vis-à-vis the model offered by them along with the offer |  |
| 5 | - Inspection: the equipment with accessories will be subject to acceptance by our divisional quality-engineering department at BEML Site. |  |