TENDER REF: 6300031534

ANNEXURE I – COMPLIANCE TO TECHNICAL SPECIFICATIONS & CONDITIONS

Description  :Supply, Install & Commissioning of VIDEO CONFERENCING Facility in the room adjacent to the Main Conference Hall at BEML Ltd, Bangalore complex..

| Pre-qualification Criteria for the OEM for Video Conferencing Equipment : |  | COMPLIED / NOT COMPLIED / REMARKS |
| --- | --- | --- |
| 1 | The OEM must have been present in the Indian market for the last 5 years |  |
| 2 | The OEM must have been serving PSU/government customers for the last 5 years. |  |
| 3 | The OEM must have been present in the Leader Quadrant of the Gartner Magic Quadrant for video end points in the last 3 available reports. |  |

| TECHNICAL INPUTS FOR DESIGN OF THE SYSTEM |  |  | COMPLIED / NOT COMPLIED / REMARKS |
| --- | --- | --- | --- |
| The proposed system must support PAL with a PTZ camera. The codec must be based on ITU standards. The VC End Point should be codec based hardware. No software based solution is accepted here. All components of the VC system like Codec, Camera and Microphone should be from the same OEM |  |  |  |
| 4 | Package | 49’ Full HD 1080p60 camera(s) with active speaker detection solution, codec, mics and remote control |  |
| 5 | Video Standards and Resolutions | It should support both H.323 and SIP standards for communications. |  |
| 6 |  | It should support H.261, H.263, H.264 AVC, H.264 SVC, H.264 High Profile/H.265 |  |
| 7 |  | It should support 1080p 60 fps, 1080p 30 fps, 720p 60 fps and 720p 30fps |  |

| Sl. No. | Description | Technical Details | COMPLIED / NOT COMPLIED / REMARKS |
| --- | --- | --- | --- |
| 1 | Content Standards and Resolutions | It should support content sharing using standard based H.239 & BFCP.It should also support audio from PC used for content sharing. |  |
| 2 |  | It should transmit both people and content both simultaneously to the far end location at 1080p 30fps |  |
| 3 | Audio Standards and Features | It should support G.711, G.728, G.729A, G.722, G.722.1, AAC-LD or better |  |
| 4 |  | It should support 20kHz or better bandwidth with crystal clear audio and stereo sound. |  |
| 5 |  | Automatic Gain Control and Automatic Noise Suppression |  |
| 6 |  | Keyboard noise reduction and instant adaptation echo cancellation |  |
| 7 | Video and Audio Inputs | 1 x HDMI/HDCI input for connecting main HD Camera |  |
| 8 |  | 1 x HDMI/HDCI input for connecting secondary HD camera |  |
| 9 |  | 1 x HDMI input for connecting PC/Laptop to share HD content |  |
| 10 |  | 1 x HDMI/Component for connecting VCR/DVD Player |  |
| 11 |  | 2 x Microphone Input (Support for minimum 4 mics). 3 mics to be supplied from day 1. |  |
|  |  | 2 x RCA line-in |  |
| Sl. No. | Description | Technical Details | COMPLIED / NOT COMPLIED / REMARKS |
| 12 | Video and Audio Outputs | 2 x HDMI output for connecting main monitor & second monitor. |  |
| 13 |  | 1 x HD output for connecting additional monitor or DVD recorder. |  |
| 14 |  | 1 x RCA pair stereo line-out |  |
| 15 | Other Interfaces | 2 x 10/100/1000 LAN port |  |
|  |  | 2 x USB for software upgrade and connecting external devices |  |
| 1 |  | 1 x RS-232 mini-DIN 8-pin or equivalent |  |
| 2 | Camera | It should support 1080p60fps with 10x optical zoom. |  |
| 3 |  | It should be capable of enhancing video conferencing experience with advanced face- recognition technology such that it automatically scans the room and seamlessly commands the main camera to appropriately frame the users during a call without any manual intervention. This feature should be available from day 1. |  |
| 4 |  | The solution must have the option to show both the active speaker and the room at the same time. |  |
| 5 |  | It should support PAN Range of +/-90°, and Tilt of +/-20° |  |
| 6 | Network Features | H.323 and SIP bandwidth up to 6 Mbps |  |
| 7 |  | IPv4 and IPv6 support from day one |  |
| 8 |  | Auto Gatekeeper Discovery |  |
| 9 |  | IP Precedence, QoS |  |
| 10 |  | H.323 based Packet Lost Recovery |  |
| 11 | Multisite Feature | The solution must support an in-built license or external solution to allow up to 8 locations, including this end point, to connect at up to 1080p resolution. This should be available from day 1. The codec must be able to natively integrate with Skype for Business Online without any additional cables |  |
| Sl. No. | Description | Technical Details | COMPLIED / NOT COMPLIED / REMARKS |
| 13 | Security | Media Encryption (H.323, SIP): AES-128, AES-256 |  |
| 14 |  | The system should also support local recording of an encrypted call by connecting an external recorder. |  |
| 15 |  | Authenticated access to admin menus, web interface and telnet API |  |
|  |  | Local account password policy configuration |  |
|  | Other Standards | H224/H.281, H.323 Annex Q, H.225, H.245, H.241, H.239, H.243, H.460 |  |
|  | Compatibility | This solution must be seamlessly compatible with the existing Polycom Real Presence Group 700 equipment. |  |
|  |  | Trolley for VC & Display mounting |  |

Requirement of Video-conference Equipment:

| Sl. No | Equipment Description | COMPLIED / NOT COMPLIED / REMARKS |
| --- | --- | --- |
| 1 | High definition Camera- As per International standard H.323 |  |
| 2 | Mic Array(2)- As per International standard H.323 |  |
| 3 | Codec/Remote Control unit-As per International standard H.323 |  |
| 4 | 4 All necessary power, networking, HDMI cables |  |
| 5 | AMC for one year |  |
| 6 | License-Multipoint-(1+7)- 1 Host + 7 other Teams |  |
| 7 | Desktop Interface for one user |  |
| 8 | Trolley for TV |  |
| 9 | 49” LED TV |  |

Technical Terms & Conditions:

| Sl. No. | Clause | COMPLIED / NOT COMPLIED / REMARKS |
| --- | --- | --- |
| 1 | Delivery : To be delivered to BEML, Bangalore Complex. |  |
| 2 | Warranty : The Video Conferencing with complete accessories shall be guaranteed for the period with 12 calendar months from the date of commissioning. During this period any part found defective should be repaired/replaced by the firm at free of cost |  |
| 3 | Commissioning: The firm should depute their competent service engineer for installation, commissioning proving the system up to entire satisfaction of BEML at free of cost. |  |
| 3 | The following publication should be supplied by the firm. - Operation instruction manual - Guarantee certificate |  |
| 4 | After Sales and Service : In case of breakdown during / after warranty, the manufacturer / authorized Dealer shall have capability to repair the equipment within 24 hours from the time of intimation. |  |
| 5 | Training : The firm has to provide operators training inside BEML after commissioning of the equipment’s. |  |

The Bidder shall comply with all the clauses of ELIGIBILITY CRITERIA. Any bid not meeting any one of the qualification requirements & clauses mentioned under Prerequisites and Terms & Conditions will not be considered for Technical & commercial bid (Price bid) for evaluation and gets rejected summarily.

All supporting documents should be uploaded in the technical bid (Collaboration folder)