SCOPE OF WORK

MANUFACTURE AND SUPPLY OF FABRICATED BOGIE FRAME/ BOLSTER            WITH FINISH PAINT FOR BROAD GAUGE DMC AND TC CARS – MEMU-3 Phase PROJECT

I.  Brief Scope of Work & Quantity  :

| Sl. No | Part / Drawing No. | Description | UOM | Qty/ DMC | Qty/ TC | Total Qty for 29.5 Rakes (59DMC+177TC) | Scope of Work |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | 84983001 | BOGIE FRAME ARRANGEMENT -DMC | NO | 2 | 0 | 118 | Supply of Fabricated DMC & TC Bogie Frame with Finish Paint |
| 2 | ET03150 | BOGIE FRAME ARRANGEMENT-TC | NO | 0 | 2 | 354 |  |
| 3 | EM04110 | BOGIE BOLSTER ARRGT.- DMC | NO | 2 | 0 | 118 | Supply of Fabricated DMC & TC Bogie Bolster with Finish Paint |
| 4 | ET04130 | BOGIE BOLSTER ARRGT - TC | NO | 0 | 2 | 354 |  |

Note DMC: Driving Motor Car with Pantograph. & TC:  Trailer Car.  (1 Rake- 2DMC+6TC)

II.  Detail Scope of Work :

- Vendor Should be approved source of RDSO/ICF.

- Vendor should follow, Manufacture & Supply Bogie Frame & Bogie Bolster (DMC & TC) as per RDSO Tech Spec. No. C-9202 (Latest Revision) and supply conforming to BEML drawings.

A. Details of Bogie Frame assembly TC & DMC

- Bogie Frame Assembly Consists of following Sub Assemblies:

- Headstock Assembly.

- Side Frame Assembly.

- Transom Assembly.

- LS Beam Assembly.

- Nose Suspension Brackets (DMC only).

- And Other Mounting Brackets.

B. Scope of Work for Manufacture of Bogie Frame/ Bogie Bolster DMC & TC

- Detail Components :

- All Components to be only fabricated by Flame Cutting/Plasma Cutting. All Edge Preparation shall be done by Machining only.

- In case Component is not Straight, Straightening of Component shall be carried out.

- Forming of Parts to be done preferably using Single Press tools and clearance to be obtained from BEML if stage wise forming tools are to be used.

- All Parts/Members shall be first degreased and derusted in a degreasing/derusting plant.

b.  Fabrication  :

- All the Parts/Members shall be welded using appropriate fixtures during assembly stages.

- Weld the straight portion of the parts/members by Submerged Arc welding & weld the Curved Contour by down hand MIG Welding using Manipulator wherever applicable.

- The assemblies shall be checked for its Dimensions & Inspected.

- The Joints between Side Frames with Transom, Headstock & Longitudinal shall be welded properly for Trailer Car Bogies   using appropriate Stagewise JIGs.

- The Joints between Side Frames with Transom, Headstock shall be welded properly for DMC Bogies using appropriate Stagewise JIGs.

- Nose Suspension Bracket shall be welded in the Transom for Motor Bogies using necessary fixtures for DMC Bogies only.

- Axle Guides shall be welded in the Bogie Side Frame with the help of   Axle Guide Welding Fixture.

- Alignment of the Axle Guide shall be checked by using Master Gauges, Straight edge & level Bed for Guide Axle .

- Any Slight Discrepancy in location can be rectified by suitable local heating of the Frame.

- Any Heat corrections at Sl. No.7 should follow Axle Guide re-alignment.

- After Proper Fixing of Guide, the dimension shall be recorded.

- Manufacturers name and Serial numbers for All Bogie Frames and Bogie Bolsters to be punched on the assemblies as locations as per drawings.

c.  Machining

- Machining has to be carried out wherever required as per drawings

- Machining of Bogie Bolster to be carried out  as  specified in Bolster Drawing No. EM04110 & ET04130.

III. Stress Reliving:

The Stress Reliving of the Bogie Frame/Bolster shall be carried out in an Oil Fired/Electric Furnace equipped with Thermocouples & Recorders.  Proper Fixtures to be used to avoid any  distortion of the Bogie/Bolster during Heat treatment. If any resetting is required to be done after Heat treatment in order to achieve required dimension the Bogie Frame/Bolster shall be again suitable heat Treated after such re-setting.(For Detail Scope for Stress Reliving refer Annexure-2 of  RDSO Tech Spec. No. C-9202 -Latest Revision).

IV.  Shot Blasting   :

- Cleaning of the Surface of the Bogie Frame, Bogie Bolster with Pressurized Air/Emery paper to remove dust & Debris.

- Apply Rust Preventive Oil on the Machined Surfaces/Tapped Holes and Mask.

- Holes, Machined Surfaces and Earthing Surfaces to be masked to avoid entry of Paint or Shots.

- Axle Box Guides shall be masked completely while Shot blasting & painting of Bogie Frame

- Shot/Abrasive blasting of Bogie Frame, Bogie Bolster to grade SA 2.5 of ISO 8501-1-1988.

- Offer for Inspection.

V.  Finish Painting of Bogie Frame, Bogie Bolster (DMC & TC)  ( Refer Drg No. 84928101 & 84928101)

- Check the Condition of Masked Areas and the Painting Equipment.

- Apply two Coats of Red Oxide Primer (ROZ) Painting within 4 Hrs of Shot Blasting to arrest Rusting and Corrosion to IS: 2074.

- Apply Synthetic Enamel Black  Under Coat Painting to IS:8662

- Apply Synthetic Enamel Black  Finish Coat Painting to IS:8662

- Cleaning of paint spill/spray over on machined surfaces/Holes/Threads.

- Offer for Inspection.

All Other General Conditions of Painting Shall be followed as per C-9202 (Latest) & Drawing No. 84928101 & 84928201

VI.  Inspection & Clearance  :

- Offer Jigs and Fixtures to assemble Sub Assemblies and Bogie Frames for Inspection

- Offer Templates for Detail Components for Inspection.

- Offer the Bogie Frame, Bogie Bolster to BEML inspection/BEML Customer inspection. Associate with BEML quality team/ RITES  for final inspection & clearance from end user and shall carryout any modifications suggested by inspection/quality team.

- Only Qualified/Certified welders shall be allotted for welding.

- The welded Joints between side frames with Transom & Headstock shall be cleared by Radio Graphic Test. Radiography test shall be carried as specified in the drawings and C-9202 by NABL Accredited Agency & Radiographic films to be submitted along with Reports to BEML.

- BEML at their sole discretion may inspect any item at any stage of Bogie Frame/ Bolster  Manufacturing.

- The Vendor shall maintain & provide a copy of test reports, radiography tests, Stress relieving/inspection reports and Check sheets to BEML/RITES.

- All Bogie Frames and Bolsters are to be packed properly in order to protect the complete assembly as well as the machined surfaces before dispatch.

VII. Delivery Schedule :

- First two Bogie Frames with Bolsters of DMC/TC shall be delivered to BEML for carrying out Static Load tests.

- After Successful completion of Static Load test of the First two Bogie Frames with Bolster & Inspection Clearance, the Vendor is requested to supply  Bogie Frames & Bolster as under :

- DMC- 4 Nos./Month &

- TC- 12 Nos./Month.

VIII. Material Scope:

1. BEML Scope.

- Raw materials in the form of Plates/Sheets.

- Rounds and Tubes in full Lengths.

- Bought Out Items as per Annexure-A

- Applicable Drawings.

- C-9202 Specification.

- Painting Specification.

- Inspection Check Sheet/Inspection Test Plan for Bogie Frame & Bogie Bolster

Vendor Scope.

- All welding consumables(BEML/RDSO Approved),Paint Materials, Oils, Lubricants, Grease etc

- Shots for Shot Blasting to grade SA 2.5 of ISO 8501-1-1988

- Material Specification for Welding consumables, Heat Resistant, Rust Preventive Oils & paints and its expiry details shall be furnished to BEML for approval before manufacturing of the first proto Bogie Frame.

LIST OF BOUGHT OUT ITEMS UNDER BEML SCOPE

| Sl No. | Part No. | Description | UOM | Qty/DMC | Qty/TC |
| --- | --- | --- | --- | --- | --- |
| Bought Outs-Bogie Frame |  |  |  |  |  |
| 1 | ET03116 | GUIDE ASSY (FOR BOGIE FRAME) | NO | 0 | 16 |
| 2 | M03094 | GUIDE WITH CLOSING PLATE | NO | 16 | 0 |
| 3 | ET03130 | BRACKET | NO | 0 | 8 |
| 4 | ET03131 | BRACKET | NO | 0 | 8 |
| Bought Outs-Bogie Bolster |  |  |  |  |  |
| 1 | ET04138 | HOUSING ASSY FOR SPIGOT-TC | NO | 0 | 4 |
| 2 | EM04114 | SPIGOT FOR BOGIE BOLSTER-DMC | NO | 4 | 0 |
| 3 | ET04120 | SPECIAL BOLT | NO | 0 | 4 |
| 4 | ET04117 | HOUSING FOR CENTER PIVOT | NO | 0 | 2 |