Annexure-B

SCOPE OF WORK

MANUFACTURE AND SUPPLY OF TC-SIDE WALL ASSEMBLIES FOR MEMU-3Phase  PROJECT:

- Brief Scope of Work & Quantity :

| Sl No | Part No. | Part Descrn | Qty/ Car | Total Quantity Reqd for 78 Cars | Scope of Work |
| --- | --- | --- | --- | --- | --- |
|  | SIDE WALL ASSY |  |  |  |  |
| 1 | ET14101SC | S/W ASSY COMP. AREA | 2 | 156 | Manufacture and Supply of TC Side Wall Assembly in unpainted condition. |
| 2 | ET14102SC | S/W ASSY COMP. AREA | 2 | 156 |  |
| 3 | ET14104SC | S/W ASSY STANDEES | 2 | 156 |  |
| 4 | ET14154SC | LAVATORY S/W L.H | 2 | 156 |  |

LIST OF SUBASSEMBLIES TO BE SUPPLIED BY VENDOR ALONG WITH SIDE WALL ASSEMLY IN LOOSE CONDITION:

| SI.NO | PART NO | DESCRIPTION | UOM | QTY/CAR | Total Qty for 78 Cars |
| --- | --- | --- | --- | --- | --- |
| 1 | 84943029 | MOUNTING DETAILS OF LAMP BRACKET ASSY | NO | 2 | 156 |
| 2 | EM14112 | BOTTOM SIDE WALL SHEET | NO | 2 | 156 |
| 3 | ET14122 | DOOR CORNER SHEET | NO | 2 | 156 |
| 4 | ET14123 | DOOR CORNER SHEET | NO | 2 | 156 |
| 5 | ET14137 | DOOR CORNER SHEET | NO | 2 | 156 |
| 6 | ET14138 | DOOR CORNER SHEET | NO | 2 | 156 |
| 7 | ET14139 | DOOR CORNER SHEET | NO | 2 | 156 |
| 8 | ET14140 | DOOR CORNER SHEET | NO | 2 | 156 |
| 9 | ET1415410 | BACK PIECE LOTAH FILLAR | NO | 2 | 156 |
| 10 | ET1415411 | BACK PIECE LOTAH FILLAR | NO | 2 | 156 |
| 11 | ET1415413 | SUPPORT FOR PANNELING | NO | 6 | 468 |
| 12 | ET1415414 | SUPPORT FOR Z MOULDING | NO | 6 | 468 |
| 13 | ET1415415 | BACK PIECE FOR THALI RACK | NO | 10 | 780 |
| 14 | ET1415418 | SUPPORT FOR THALI RACK | NO | 2 | 156 |
| 15 | ET1415421 | BACK PIECE FOR MOULDING | NO | 4 | 312 |
| 16 | ET1415423 | BACK PIECE FOR MOULDING AROUND EXHAUST CUTOUT | NO | 4 | 312 |
| 17 | ET14157 | BOTTOM SIDE WALL SHEET | NO | 2 | 156 |
| 18 | ET14160 | BOTTOM SIDE WALL SHEET | NO | 4 | 312 |
| 19 | ET14161 | BOTTOM SIDE WALL SHEET | NO | 2 | 156 |
| 20 | ET14162 | BOTTOM SIDE WALL SHEET | NO | 2 | 156 |
| 21 | ET14163 | BOTTOM SIDE WALL SHEET | NO | 4 | 312 |

Note :

- Rake Formation: 1 Rake- 2DMC+2TC1+2TC2+2TC3

- Side walls to be supplied as a set of one Car

- Vendor should be approved source of RDSO/ICF/RCF

- Manufacture & Supply of Aggregates should conform to Latest drawings and to the material specification/Standards/Test, indicated in the respective drawings is to be adhered/ensured.

- Firm to supply aggregates for one TC Car to ascertain the fitment/installation aspects before bulk supply.

II :  DETAILED  SCOPE OF WORK:

- Details of  TC Side wall Assy

- Side Wall Consists of following  Major Sub Assemblies:

- Side wall sheet Assemblies

- Body Pillar Assemblies.

- Side Wall Structure

- Door Corner Sheet Assemblies

- Manufacturing of Detailed Components:

- All Components to be only manufactured by CNC Flame Cutting/Plasma Cutting/Laser Cutting/Shearing for Plates & Sheets, Saw Cutting for Tubes/Pipes/Bars/Rods. Vendor shall submit a nesting plan to BEML for Approval before taking up for manufacturing.

- Dress, Grind and Deburr cut edges of all detailed components and all edge preparation to be carried out by beveling.

- Straightening of Components to be carried out wherever required before taking up for forming/ Assembly.

- All Plasma cut/Flame cut/Laser cut components to be shot blasted before taking up for forming/Assembly. All Parts/Members shall be first degreased and derusted and Paint with Weld primer

- Forming of Parts to be done without tool marks

- Mass production of detailed components can be taken up only after getting acceptance from BEML for first batch/Proto Assy Clearance.

- Fabrication of Subassemblies/Assemblies:

- All subassemblies/Assemblies to be fabricated out of weld jigs in order to achieve uniformity in dimensions, quality of above production. This Jigs to be shown to BEML/SIT during manufacturing Proto and subsequent production.

- Welding of Fabrication Structures to be carried out as specified in drawings.

- Welding shall be carried out uniformly along the length to avoid distortion.  During Fabrication, Weld defects, Weld spatters, heat marks etc due to welding shall be removed before taking up for next assembly. Welder should be qualified for the requirements of IS: 7310 and IS: 817 or any equivalent international standards.

- Pickling and Passivation to be carried out for removing weld marks on Stainless Steel surfaces.

- Fabricate all major Sub Assemblies viz, Side wall Sheet Assembly, Body Pillar Assembly, Side wall Structures, Door Corner sheet Assemblies etc., as per Drawings.

- Dimensional check in accordance with the check sheet shall be carried for all major Sub-assemblies before taking for Major Assembly and report to be submitted to BEML.

- Loading of Sub assemblies viz, Side wall Sheet assembly, Side wall Structures on Main Jig as per drawing, after welding of structure with skinning, other details like Window Sealing arrangement, waist Rails, Light Rail, Horizontal Stiffeners etc to be welded.

- Cleaning of the Surface of the Side wall with Pressurized Air/Emery paper to remove dust & Debris.

- Inner side of Side wall Assembly and Intermediate parts to be cold phosphate and Outside skin should be Degreased and De rusted.

- Skin tensioning of the Side Wall shall be so carried out that variation from profile of maximum 1 mm per meter length of the Side Wall is achieved.

- The Side wall assembly shall be checked for its Dimensions after Side wall structure stage and after completion of Full assembly, in accordance with check sheets and reports to be submitted to BEML Inspection Team.

- All other dimensions / standards / specifications to be maintained / followed for all Subassemblies / Assemblies as per respective Drawings with latest alterations. Before starting the production, vendor should ensure to manufacture as per the latest drawing as and when it is updated.

III, INSPECTION & CLEARANCE :

- Proto Inspection :

- Offer the   Sub Assemblies Viz, Side Wall Structure, Side wall Skin Assembly and Door Corner Sheet assemblies etc, in Tack Welded Condition to BEML/SIT/RITES inspection.

- Offer Side wall assembly to BEML/SIT/RITES Inspection in Jig Loading stage of Side wall structure, Tack Welded Condition, after Loading Side wall Structure on Side wall Skin.

- Offer the completed Proto Side wall to BEML/SIT/RITES inspection. Associate with BEML quality team for final inspection & clearance. Firm shall carryout any modifications suggested by BEML/SIT/RITES Inspection team without any additional cost.

- General Inspection Conditions:

- Offer All Side wall assemblies to BEML/SIT/RITES inspection. Associate with BEML/SIT/RITES Inspection team for final inspection & clearance and firm shall carryout any modifications suggested by BEML/SIT/RITES inspection team.

- The Construction throughout Side Wall Assemblies shall be absolutely Leak Proof. All Side wall welded joints shall be tested after Side wall Assembly by Applying Chalk Powder on one side and Kerosene oil on other side. Dye Penetration Test to be carried out for all Side Wall weld joints as and when insisted by BEML/SIT/RITES inspection team at their discretion.

- Chemical and Mechanical testing of samples should be carried out as and when insisted by BEML/SIT/RITES inspection team at their discretion

- BEML/SIT/RITES at their sole discretion may inspect any item at any stage of Manufacturing of Side wall Assembly.

- The Vendor shall maintain & provide a copy of test reports, inspection reports and Check sheets to BEML Inspection Team. All the witnessed inspection reports like kerosene oil leak test, dye penetrate testing, chemical & Mechanical testing, Dimensional report etc. should be submitted to BEML in digital form via email or CD/DVD.

IV. Delivery Schedule :

- Proto TC Side Wall Assemblies shall be delivered to BEML at RC-II Unit/ Bangalore Complex, after Inspection Clearance & after implementing any modifications if any suggested by BEML/SIT/RITES.

- After Successful Installation of Proto Side Wall Assemblies at BEML, Bulk Production to be taken up after obtaining Clearance from BEML  Inspection Team.

- TC Side Wall Assemblies - 12 Cars/Month (After Proto Clearance).

- All aggregates to be supplied as a set in Car wise using proper holding Jigs to RC-II Unit, KGF/Bangalore Complex based on confirmation by M/s.BEML.

IV. Material Scope:

- BEML Scope :

- Raw materials in the form of Plates/Sheets.

- Rounds and Tubes in full Lengths.

- Applicable Drawings.

- Inspection Check Sheet/Inspection Test Plan for Side wall Assembly, Side wall Skinning Assemblies, Door Corner Sheet Assemblies etc.,

- Vendor Scope.

- All required welding consumables, Rust Preventive oils, Lubricants as specified in Drawing/RDSO Specifications.

- Material Specification of Welding consumables, Rust Preventive Oils, Derusting/ Degreasing/Cold Phosphate consumables & paints and its expiry details shall be furnished to BEML for approval before manufacturing of the first proto Side Wall Assembly.

V. General Notes to Comply:

- Quality Assurance Plan (QAP) should be submitted by firm for approval.

- Process Qualification Record (PQR)/Welding Procedure Specification (WPS) should be available.

- Vendor should be ISO: 9001 – 2015 Compliant.

- Before taking up manufacturing activity, Self-Inspect the M/s BEML supplied component for transit damages, In case of any defects same has to be communicated with detailed report and send back the same to BEML in as is condition.

- Vendor shall submit nesting plan to BEML for Approval before taking up for manufacturing. Off cuts and Scrap generated during manufacturing of components to be returned to BEML as per approved nesting plan.

- Ensure components supplied by M/s.BEML and Fabricates Subassemblies and Assemblies are stored properly and damages due to poor & improper storage, workmanship etc. should be the responsibility of Vendor.

- Vendor shall ensure the proper fitment of aggregates while shell integration/furnishing at initial batches of supply & further in case of any fitment issues at RC-II unit KGF and Bangalore Complex. Vendor has to engage their representative for necessary correction and implementation of those corrections in further supplies.

- Any minor scope Changes/Modifications in drawing or during inspection if any to be taken up by vendor without any additional cost.

- Ensure all side wall assemblies to be properly Stored/Protected/Covered to avoid Rusting and Damages at vendor premises and also to supply to BEML in Rust Free condition.

- All Side wall assemblies to be properly protected to avoid damages during loading, unloading and transportation.