### Request for Quotation

#### (Part-B)

#### For supply of

#### " CONSTRUCTION & MAKING OF FOUNDATION BED FOR 500 KVA DG SET.”

#### Tender No. 6300033938

#### Tender Closing date:31.10.2020, 16:00 Hours

#### To

## BEML Limited

Vendor’s Time and Expenses: To be borne by the vendor

No representation would be entertained on any errors if found in the RFQ. However vendors to bring such errors / omissions to the notice of BEML for necessary corrective action

##### Exhibit – A

|  | Tentative requirement of CONSTRUCTION & MAKING OF FOUNDATION BED FOR 500 KVA DG SET.- Beml Limited., Bangalore Complex. Division: BEML, Bangalore Complex |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Requirement Specification |  |  |  |  |
| Sl. No. |  |  | Item Name | UOM | Quantity |
| 1 |  |  | Dismantling of existing concrete floor Dismantling the existing cement concrete flooring of required size and depth and removing the dismantled material from the site. | AU | 1 |
| 2 |  |  | Earth work excavation for foundation bed Earth work excavation over areas for foundation bed not existing 1.50mtr depth and getting out the excavated earth from the site etc., complete. | AU | 1 |
| 3 |  |  | Fabrication &fixing of reinforcement RCC Fabricating and fixing reinforcement for R.C.C. works including cutting, straightening, bending and placing in position at all levels & binding with approved gauge binding wire. Note: Steel will be supplied by BEML | AU | 1 |
| 4 |  |  | Supply & laying in position RCC Providing and laying in position Reinforced cement concrete work of grade 1:1.5:3 (1 cement: 1.5 coarse sand: 3 graded stone aggregate 20 mm nominal size) using graded hard granite stone aggregates of maximum 20mm nominal (downgraded) | AU | 1 |
| 5 |  |  | Centering and shuttering incl. strutting Centering and shuttering including strutting, propping etc., and removal of frame work | AU | 1 |
| 6 |  |  | Sup &filling sand allround the found bed Supplying and filling sand all round the foundation bed of required thickness all as specified and directed. | AU | 1 |
| 7 |  |  | Rendering in CM1:6, 20mm thick surfaces Rendering in CM 1:6, 20mm thick to the concrete surfaces finishing the surfaces even and smooth including curing, tools, tackles etc., complete all as specified and directed | AU | 1 |
| 8 |  |  | Removal of debris and dumping Removing the debris from the site, transporting outside factory and dumping at a designated place shown in township | AU | 1 |
|  | NOTE:L1 status will be considered as 1 AU for CONSTRUCTION & MAKING OF FOUNDATION BED FOR 500 KVA DG SET - Beml Limited., Bangalore Complex. Note: - The tendering process is E-Mode - You should have Digital Signature Certificate (class III) issued by authorized agencies to submit commercial bid on E-mode. - Please open the collaboration folder to view Tender Documents Kindly go through all the tender documents and bid accordingly. - To be delivered to BEML Bangalore Complex on FOR basis. - Quotation should be valid for 120 days from date of tender opening. - FAX/E-mail/Manual quotations are not acceptable and quotations received through these modes will be rejected. Incomplete and conditional offers are also liable for rejection. - If any of the vendors have defaulted in supplies/services to BEML limited after placement of purchase order or If any of the vendors have backed out from their quotation submitted for the enquires of BEML limited or If any of the vendors have given wrong Information/documents to BEML Limited along with quotations, their quotation will not be considered and their quotation will be rejected. The quotations of the Firms who have not provided Service during the Warranty period of any product supplied by them to BEML Ltd will not be considered & will be rejected. - Corrigendum if any will be updated in BEML website only. - For any technical difficulty in submitting the Bid, please contact Mr.KrishnaMohan in the following numbers: 080-22963269 / 22963141. - Any clarification in the tender, please contact Mr. UDAY KUMAR in 080-25348770 / 25022638/25022635, E-Mail ID: rmm-1@beml.co.in Delivery : February 2020 |  |  |  |  |