Annexure-1

TECHNICAL SPECIFICATION OF MECHANICAL SHAFT SEAL FOR 5/8” PUMP SHAFT

(CONICAL SPRING TYPE)

Pump drive shaft side

Application:

Shaft diameter		 : 5/8” (15.875 mm)

Shaft speed 		: 1420 rpm

Working pressure 	: 10 bar

Oil Temperature 	: 176°C (Max.)

Rotation		: Anticlockwise (viewed from pump drive shaft)

Dimensions:																							L1(working length) 	: 20~23 mm

L4 			: 6~7 mm

D1 			: 5/8 inch (15.875 mm)

D3 			: 27 mm (max.)

D7			: 29 mm (max.)

Material:																							Rotating Face	 	: Silicon Carbide, Tungsten carbide

Stationary Face	: Silicon Carbide, Tungsten carbide

Spring Material &	: Stainless Steel (SS316)

Metal Parts

Flexible Seal Material	: Fluorocarbon Elastomer (FKM) /

Fluorocarbon + PTFE - encapsulated